Edge AI Camera Monitor & Motion Security Guard

- What it does: Connects to a webcam or network security camera, processes the feed in real-time, detects movement, filters out noise (like shadows or tree branches), saves video clips when motion is detected, and sends alerts.

- Why it's useful & important: A practical home or office security application. It runs locally without needing expensive cloud servers.

- Core Tech: OpenCV image filters, video encoding (integrating FFmpeg to write H.264 MP4 files), background subtraction algorithms, email/notification dispatchers.

Added more features

### 1. Cybernetic Target Acquisition HUD (Terminator Vision)

- The Concept: Instead of drawing standard boring green boxes around motion, render a futuristic HUD overlay on the video feed.

- The Features:

  - Dynamic Crosshairs: Reticles that lock onto moving objects and track them.

  - Target Classification Tags: HUD text showing calculated metrics: [SPEED: 1.2 m/s], [COORDINATES: X: 412, Y: 243], [THREAT LEVEL: LOW/MEDIUM/HIGH].

  - Diagnostic Sidebars: Rolling terminal-style statistics on the left/right of the frame (CPU load, frame processing latency, memory usage, security status).

### 👥 2. Biometric "Friend-or-Foe" (IFF) Identification

- The Concept: Integrate a lightweight, offline facial recognition system to categorize people entering the camera's view.

- The Features:

  - Authorization Matching: If the camera recognizes your face, the HUD frame glows soft blue/green and displays [STATUS: AUTHORIZED - MEJBA].

  - Intruder Alert: If an unrecognized face is detected, the HUD flashes warning borders, locks a red reticle on them, and displays [STATUS: UNIDENTIFIED INTRUDER].

### 🔊 3. Synthetic Voice Response System (Robotic AI Voice)

- The Concept: When an event occurs, the monitor talks back to you in a localized, synthesized robotic voice.

- The Features:

  - Intruder Warning: "Warning. Unidentified activity detected in Sector Alpha. Initiating recording protocols."

  - Status Updates: "System initialized. Diagnostics nominal. Sentinel online."

### 📡 4. ASCII Sonar Radar Sweep (Terminal Interface)

- The Concept: Beside your video window, keep a running terminal display that shows a 2D radar screen sweeping back and forth.

- The Features:

  - Object Blips: Whenever the camera detects motion, a bright blip appears on the radar grid at the corresponding angle and distance, slowly fading away as the radar sweeps past it (like a submarine sonar).

  - Emergent Alerts: The terminal rings high-frequency cybernetic chirps when a new blip appears.

### 🧠 5. Anomalous Behavior Analytics (Self-Learning AI)

- The Concept: Instead of simple motion triggers (which get set off by shadows or wind), the engine learns what is "normal" in your room (e.g. you sitting at your desk, the fan spinning).

- The Features:

  - Habit Baseline: Builds a local statistical model of daily movement patterns.

  - Anomaly Trigger: If something unusual happens (e.g., someone walks past the desk at 3:00 AM, or someone falls over), it flags it as a "behavioral anomaly" and starts recording.

# Implementation Plan - Project: Cyber Sentinel (Edge AI Monitor)

An advanced, edge-computing C++ security application that transforms a standard webcam feed into a cybernetic target acquisition display, with real-time motion detection, neural-network-based biometric face recognition (Friend/Foe), and a terminal-based ASCII sonar radar.

## Architectural Overview

The application will run on a multi-threaded architecture to ensure real-time performance (30+ FPS) on standard PC hardware:

- Thread 1: Main Feed & CV Pipeline: Captures video frames, runs background subtraction for motion detection, executes DNN-based face detection/recognition, renders the cybernetic HUD, and manages window display.

- Thread 2: Sonar Radar Console: Computes and prints a sweeping 2D ASCII radar screen in the command terminal, mapping detected targets from the camera field of view.

- Thread 3: Alert & Recording Engine: Handles file writes (asynchronously writing H.264 MP4 threat video clips) and fires system alerts without blocking the video capture loop.

## Technical Specifications & Dependencies

### Build Environment

- Operating System: Windows

- Compiler: Microsoft Visual C++ Compiler (MSVC 2019 Build Tools, C++17)

- Build System: CMake (via MSVC build toolchain)

- Primary Library: OpenCV 4.9.0 (Prebuilt Windows package, downloaded and linked via CMake)

- Deep Learning Models: OpenCV DNN loading pre-trained ONNX models:

  - Face Detection: YuNet (ultra-fast edge face detector)

  - Face Recognition: ArcFace / MobileFaceNet (for generating facial embeddings)

## Proposed Changes & Files

We will initialize a new CMake C++ project in a subdirectory under the scratch path: C:/Users/mejba/.gemini/antigravity/scratch/cyber_sentinel/

### Project Files

#### [NEW] CMakeLists.txt

Defines project build configurations, links OpenCV libraries, and specifies include headers.

#### [NEW] src/main.cpp

Main application entry point. Handles program initialization, OpenCV window creation, thread synchronization, and the main video capture loop.

#### [NEW] src/detector.hpp & src/detector.cpp

Contains the MotionDetector class (handles background subtraction, thresholding, noise filtering, contour grouping) and the BiometricClassifier class (loads DNN models, computes facial embeddings, and calculates L2 distance thresholds for friend-or-foe recognition).

#### [NEW] src/hud.hpp & src/hud.cpp

Implements drawing routines to overlay neon UI grids, scanning lines, lock-on target boxes, and sliding system stats directly onto video frames.

#### [NEW] src/radar.hpp & src/radar.cpp

Runs the console-based 2D sonar grid thread, printing sweeping lines, degree metrics, and lingering dots representing target positions.

#### [NEW] src/alert.hpp & src/alert.cpp

Manages saving H.264 video clips of security violations and triggering Windows notification bubbles.

#### [NEW] run.bat

A quick-compile-and-run shell script that downloads OpenCV 4.9.0, configures CMake, builds the executable using MSBuild, and executes the system.

## Detailed Component Design

### 1. Cybernetic HUD (Terminator-Style UI)

- Target Lock reticles: Draws bracket corners around objects that dynamically scale. If target threat increases, color shifts from cyan to warning red.

- Scanning Text overlay: Scrolling telemetry on the side of the window displaying camera feed diagnostics (resolution, FPS, RAM utilization, threat index).

- Grid layout: A subtle coordinate grid with horizontal crosshairs center-aligned on screen.

### 2. Biometric Face Classifier (Friend/Foe DNN)

- Face Detection: Uses OpenCV DNN with YuNet, which processes an image in a few milliseconds on standard CPU.

- Face Recognition: ArcFace ONNX model maps faces to a 128-dimensional vector space.

- Database: A simple local directory containing face images of authorized users (e.g. database/mejba.jpg). On startup, these images are loaded, mapped to vectors, and cached.

- Inference: When a face is detected in the video stream, its embedding vector is compared against the database using Euclidean (L2) distance. If the distance is below the threshold, it identifies the user as FRIEND: [Name]. If not, it marks them as UNAUTHORIZED FOE.

### 3. Sonar Radar Terminal UI

- We will use standard Windows Console control commands to manipulate the cursor position without clearing the screen, preventing terminal flicker.

- The radar draws a circular sweep line using coordinates. Targets are mapped using camera contour centers relative to screen coordinates.

- It displays tracking dots that fade in intensity as the sweep rotates past them.

## Verification Plan

### Automated Build Verification

- Compile testing via run.bat using MSBuild to ensure error-free compilation.

- Verify CMake locates the downloaded prebuilt OpenCV installation.

### Manual Verification

- Webcam Test: Run the executable to verify the camera starts and streams video.

- HUD Verification: Wave hand in front of the camera and verify:

  - Neon bounding boxes acquire and track the hand.

  - Velocity lines stretch in the direction of motion.

- Biometrics Verification:

  - Point the camera at an authorized face (registered in database/) and check if the HUD highlights in green with the name.

  - Point at an unknown face (or photo) and check if the HUD flashes red with UNAUTHORIZED FOE alerts.

- Radar Sweep: Check the command prompt window to verify the sonar radar sweeps smoothly at 30+ updates per second, highlighting blips.

- Recording Test: Trigger a "Foe" alert and verify a .mp4 file is generated inside the project folder and plays back correctly.